___________

GoMMC guide

___________

GoMMC is powerful, and there is a lot to tell. Perhaps too much to do it all in

one go. Therefore, this guide has a 'basic' and a 'advanced' section.

The basic section attempts to describe basic use of GoMMC only, and hold back

on details. If you're a first time user of GoMMC, or an advanced user looking

for only basic information, read this section.

The advanced section provides systematic, detailed descriptions of all aspects

of GoMMC. It's there if you're ready for more, or need to know more details.

=========

=============

=================

Basic section

=================

=============

=========

//

//

// Introduction

//

//

GoMMC is an MMC (MultiMedia Card) interface for the Acorn BBC B, BBC B+, BBC

Master and Electron.

It acts like a big jukebox, filled with floppies, harddiscs and tapes. Just

select one of them (by name, with a star command), and then use it as if it

were an actual, physical floppy, harddisc or tape.

The floppies, harddiscs and tapes are all stored on the MMC. Their total number

(and size) is only limited by the capacity of the MMC. For example, on a 2 GB

MMC, you would be able to store 5000 double-sided DFS floppies (400 KB each)

and still have room for 2 ADFS harddiscs of 50 MB each.

You can use the floppies, harddiscs and tapes on the MMC via the same filing

systems you used before for their physical counterparts. Although you do have

to use 'patched' versions of these filing systems, which have been fooled into

thinking they're accessing a physical floppy, harddisc or tape, while they're

actually accessing its image on the MMC. GoMMC supports (i.e. provides patches

for) a large number of different filing systems, among which many Acorn DFSes

and ADFSes, and some 'foreign' ones.

GoMMC is completely self-contained, and can make do with a 'bare' machine. It

doesn't need any actual floppy or harddisc hardware (or software) to be present

in the machine. It can fully replace all the floppy drives, harddiscs, floppies

and tapes that you have, or always wanted to have. It is also very much faster,

quieter, and easier to use, among other things.

//

//

// Using GoMMC

//

//

Basic use of GoMMC requires four main ingredients :

1) Information.

2) Installation.

3) Getting discs onto the MMC.

4) Running patched filing systems.

//

//

// Information

//

//

Take note of 'GoMMC central' on Internet. This is at URL :

http://web.inter.nl.net/users/J.Kortink/home/hardware/gommc/index.htm

Bookmark it ! And browse (if only casually) through all the pages.

//

//

// Installation

//

//

--------------------------------

Step 1 : Physical installation

--------------------------------

Go to 'GoMMC central' on Internet, and follow 'Installation guide'.

And follow the instructions.

When you've finished physical installation, a brief checkup can be performed :

- Power down the machine.

- Remove the MMC plugged into GoMMC (if any).

- Power up the machine. The top of the screen should say :

GoMMC : MMC doesn't respond

- Enter '*MMCInfo'. It should display something like :

GoMMC x.xx (day month year)

No MMC

So far so good ! Now, if you have an MMC :

- Power down the machine.

- Plug the MMC into GoMMC.

- Power up the machine.

- Enter '*MMCInfo'. This should now at least show the capacity of the MMC. If

the MMC is already (GoMMC-)formatted, it should also show the free and used

space, and the number of objects.

Note that powering down before inserting or removing the MMC is not strictly

necessary. But if you don't, always press CTRL-BREAK afterwards.

--------------------------------------

Step 2 : Download the GoMMC software

--------------------------------------

Go to 'GoMMC central' on Internet, and follow 'Software updates'.

And download the .zip file containing the latest version of the GoMMC software.

Note that new versions of the software will only appear on this page, and will

only be announced on the GoMMC mailing list, at :

http://groups.yahoo.com/group/gommc

It's highly recommended to subscribe to the GoMMC mailing list if you want to

follow developments, ask questions and/or discuss GoMMC related topics.

--------------------------------------------

Step 3 : Format your MMC (MultiMedia Card)

--------------------------------------------

Before you can use your MMC with GoMMC, you must format it with MMCFO. If you

bought the MMC from me directly, most likely it will already be formatted, so

you can skip ahead to step 4. Otherwise :

Power down, insert your MMC into GoMMC, and power up. Then run GoMMC tool

'MMCFO'. Refer to 'Running GoMMC tools' for details.

You should see something similar to :

Formatting MMC ... done

Please hard-reset your machine now

Do so, e.g. press CTRL-BREAK.

----------------------------------------

Step 4 : Verify that your MMC works ok

----------------------------------------

Enter '*MMCInfo' and verify the logic of what it displays (8704 bytes will be

'used', the other values follow from that and the MMC's size).

If it says 'Not formatted', you have not (succesfully) formatted your MMC, and

your MMC /may/ be incompatible with GoMMC (but do try step 3 again first, then

if it keeps failing, contact me).

------------------------------------------------

Step 5 : Update/refresh the GoMMC ROM software

------------------------------------------------

Run GoMMC tool 'MMCUP' and answer the questions (if any). Refer to 'Running

GoMMC tools' for details.

It is not strictly necessary to run MMCUP if it contains the same version of

the GoMMC software that you are already running (as displayed with *MMCInfo),

but if you've just received your GoMMC it is nevertheless recommended.

//

//

// Getting discs onto the MMC

//

//

To add tapes to the MMC, refer to 'Tapes' in the advanced part of this guide.

Discs (floppies, harddiscs) can be put on the MMC in two main ways :

1) Via the machine which has GoMMC installed

--------------------------------------------

This must have an actual, physical floppy and/or hard drive installed.

You will be running tools MMCADDG (to add DFS discs), or MMCADAG (to add ADFS

discs). Refer to 'Running GoMMC tools' for details about running GoMMC tools.

Note that running the tool via the MMC (i.e. the *MMCTool command) will be the

most convenient option in this case (e.g. if you have one floppy drive and no

harddisc, you would first have to put the tool on every floppy you add to the

MMC, because the tool does not allow you to swap discs).

To add a floppy disc to the MMC, insert it into your floppy drive. Then run the

appropriate tool, e.g. (specify the name you want the disc to have instead of

<your disc name>, using up to 48 characters, with no spaces) :

MMCADDG $1$ <your disc name>

if it is a single-sided, 200 KB, DFS disc, or

MMCADAG $1$ <your disc name>

if it is a single-sided, 320 KB, ADFS disc, or

MMCADDG $2$ <your disc name>

if it is a double-sided, 400 KB, DFS disc, or

MMCADAG $2$ <your disc name>

if it is a double-sided, 640 KB, ADFS disc, or

MMCADDG $3$ <your disc name>

if it is a twin (i.e. two floppies in two separate drives !) double-sided, 400

KB, DFS disc, or

MMCADAG $3$ <your disc name>

if it is a twin (i.e. two floppies in two separate drives !) double-sided, 640

KB, ADFS disc.

To add a hard disc to the MMC, use :

MMCADAG $4$ <your disc name>

If you happen to have (floppy/hard) disc images in files (e.g. if you're

running 65Link and they're on your host machine's harddisc), you can use :

MMCADDG <disc image file name> <your disc name>

for DFS disc images, or (for ADFS disc images)

MMCADAG <disc image file name> <your disc name>

if they are not 'track interleaved' (they almost never are, otherwise, use

tools MMCADDI and MMCADAI respectively).

2) Via a PC with a flash card reader

------------------------------------

This must be running Windows XP or (x86) Linux.

You will be running the utility program GoMMCio. Refer to 'Running GoMMCio' for

general information about running GoMMCio.

Your (floppy/hard) discs will have to be available as disc image files (e.g.

.ssd or .adf files as downloadable from Internet).

To add a disc to the MMC, use (refer to 'Running GoMMCio' for what to fill in

instead of 'Q') :

GoMMCio -dQ -c "MMCADDG <disc image file name> <your disc name>"

for DFS disc images, or (for ADFS disc images)

GoMMCio -dQ -c "MMCADAG <disc image file name> <your disc name>"

if the disc image file is not 'track interleaved' (it almost never is,

otherwise, use commands MMCADDI and MMCADAI respectively).

Note that if you want to add more than one disc in one go, put the commands you

would have put between the double quotes in a plain text file, call it (e.g.)

'script', and use :

GoMMCio -dQ script

instead.

Note that you can create 'empty' DFS or ADFS discs of practically any size as

well, by using GoMMCio's 'NEWDFS' and 'NEWADFS' commands (for details, refer to

'Tools : GoMMCio.exe' in the advanced section of this guide).

//

//

// Running patched filing systems

//

//

A patched filing system (an original filing system patched for GoMMC) is needed

to access the discs on the MMC. Accessing DFS discs requires a DFS type filing

system, accessing ADFS discs requires an ADFS type filing system.

The filing systems that GoMMC supports are listed under 'Supported filing

systems' in the advanced section of this guide.

To obtain the filing system ROM image (containing either an original or patched

version of the filing system), visit :

http://web.inter.nl.net/users/J.Kortink/home/hardware/gommc/fs

The first character of the filename, 'O' or 'P', indicates original or patched

respectively. The second, 'D' or 'A', indicates 'DFS' or 'ADFS' respectively.

The number that ends the filename indicates the version of the filing system.

E.g. 'PAFS130.BIN' is the patched version of Acorn ADFS 1.30.

You can introduce a patched filing system in two ways. Via sideways ROM, or via

sideways RAM.

1) Via sideways ROM

You simply program the 'P' image of the filing system into an EPROM and insert

that into a free sideways ROM socket (this can only be done with filing systems

that don't need to run from sideways RAM).

2) Via sideways RAM

You auto-load the patched filing system into sideways RAM via GoMMC (this also

takes care of resetting the machine for you, so it almost seems like the filing

system was in ROM).

First you add it to the MMC, by :

a) running tool MMCAFS. Refer to 'Running GoMMC tools' for more details. E.g.

use 'MMCAFS PAFS130' (after copying PAFS130.BIN as PAFS130 onto a DFS or ADFS

floppy).

b) running GoMMCio and using the MMCAFS command. Refer to 'Running GoMMCio' for

more details. In the command window, enter 'GoMMCio -dQ -c "MMCAFS <patched

filing system image> MMCFSx', where 'x' is the GoMMC internal filing system

number (see 'Supported filing systems' in the advanced section of this guide).

E.g. 'GoMMCio -dQ -c "MMCAFS PAFS130.BIN MMCFS3"'.

Then, on your GoMMC-equipped machine, enter '*MMCConfig FSNR x' (where x is,

again, the GoMMC internal filing system number), and '*MMCConfig FSRM y' (where

y is the number of the sideways RAM bank that the filing system will be loaded

into).

Note ! Refer to 'Sideways ROM/RAM' in the advanced section of this guide for

details about sideways ROM/RAM banks available on your particular machine.

//

//

// Running GoMMC tools

//

//

GoMMC tools (the main ones being MMCUP, MMCWI, MMCFO, MMCATO, MMCAFS, MMCADDG,

MMCADAG, MMCADDI and MMCADAI) all need to run 'natively'. That is, on your BBC

or Electron. You will find them as separate files among the GoMMC software.

You will have downloaded the GoMMC software via Internet, so the GoMMC tool

files will initially be on your machine connected to Internet (or on some other

PC you subsequently copied them to). That machine will be referred to as 'the

foreign machine'.

Note : before you run any tool file on your BBC or Electron, ensure its load-

and execution-address is (hex, i.e. &) 2000 ! Check this with '*Info <tool file

name>'. Correct them if necessary, for example by '*Load <tool file name> 2000'

followed by '*Save <tool file name> 2000+xxxx 2000' (where xxxx is the file's

(hex) size as obtained via *Info).

There are a number of ways of running a tool :

1) Via floppy

-------------

You must have floppy disc hardware installed on your (Acorn) machine, and you

must find some way to transfer the tool file from the foreign machine to a DFS

or ADFS floppy that your (Acorn) machine can read.

This can be a complicated matter, since not all modern PCs can handle (and thus

write files to) DFS or ADFS formatted floppies. They certainly can't do it 'out

of the box', software-wise.

You may want to consider using the following tools (find them on Internet via a

search engine) :

- OmniFlop (e.g. search for 'Omniflop')

- DFS or ADFS Explorer (e.g. search for 'DFS Explorer')

- XFer (e.g. search for 'XFer BBC')

Once the tool file is on floppy, to run the tool, simply type e.g. '*Run

MMCUP'.

2) Via ToolROM

--------------

You must have a 'ToolROM' (which is an EPROM containing the ROM image file

'TOOLROM', which you'll find among the GoMMC software). This contains a few

tools, like MMCUP, MMCWI and MMCFO (but not all of them).

Power down, insert the ToolROM into a free sideways ROM socket in your machine,

then power up again. Enter '*ROM' (to select the ROM filing system), then enter

e.g. '*Run MMCUP'. Enter '*Opt 1 2' and '*Cat' to see which tools are in the

ToolROM.

For more information refer to 'Tools : TOOLROM' in the advanced part of this

guide. Read this carefully, in particular take special note of the caveats !

3) Via 65Link

-------------

65Link is a 'remote filing system' for the BBC. Setting up a 65Link system is

not trivial (it also includes making a special cable), but you will be able to

run tool files straight from the harddisc of your foreign machine.

For more information, see :

http://web.inter.nl.net/users/J.Kortink/home/software/65link/index.htm

Once the tool file is on a 'disc' in a 65Link 'volume', to run the tool, simply

type e.g. '*Run MMCUP'.

4) Via the MMC

--------------

You need to use GoMMCio. Refer to 'Running GoMMCio' for details.

In the command window, enter :

GoMMCio -dQ -c "MMCATO <tool name> <tool name>"

where Q points to the MMC, and <tool name> is the name of the tool, e.g.

GoMMCio -dC -c "MMCATO MMCWI MMCWI"

or (under Linux)

GoMMCio -d/dev/sdc -c "MMCATO MMCWI MMCWI"

and verify that GoMMCio exits without error.

If your MMC is not yet (GoMMC-)formatted, add '-f -x ' (note the space) before

the '-c'. E.g. this is useful if you can only run MMCFO this way (via the MMC),

presenting (as it first seems) a chicken and egg problem.

Now power down your (Acorn) machine. Move the MMC from your flash card reader

to GoMMC. Power up your (Acorn) machine. Then enter e.g. '*MMCTool MMCWI'.

//

//

// Running GoMMCio

//

//

GoMMCio runs on a PC. You will find it among the GoMMC software. GoMMCio.exe is

for PCs running Windows XP, GoMMCio.unx (rename it to GoMMCio and ensure it has

'execute' access) is for PCs running (x86) Linux.

I.e. you need to have a flash card reader attached to a PC running Windows XP

or (x86) Linux. If you don't have such a setup, it is highly recommended to get

(access to) one. Otherwise, you won't be able to use GoMMCio, which is pretty

much essential to use GoMMC comfortably.

Remove any inserted flash cards from your flash card reader. Connect your flash

card reader to your PC. Insert your MMC. If you run Windows XP, new 'drives'

will pop up in your 'My Computer' window, usually 4 or more, each representing

a slot on your flash card reader. Find the MMC by double-clicking on every new

drive icon, and picking the one *not* complaining 'Please insert a disk ...'

(but something like '... not formatted' instead). Note the drive character as

'Q' (see below). If you run Linux, you will have to find out which device in

/dev/ represents the MMC (e.g. /dev/sdc), and note that as 'Q' instead.

Open a command window (e.g., under Windows XP, by clicking 'Start', then 'Run

...', and entering 'Command', or, under Linux, bu opening a terminal window).

Go ('cd') to the directory containing GoMMCio (e.g. 'cd \gommc').

You are now ready to run GoMMCio. Test it, e.g. by entering :

GoMMCio -dQ -l -c "#"

which will list the contents of your MMC.

============

================

====================

Advanced section

====================

================

============

//

//

// The hardware

//

//

There are a few important things to note about the GoMMC hardware before you

use the software.

Most importantly this : GoMMC has two on-board 16 KB ROM banks, one of which

contains the GoMMC software (as programmed by MMCUP), and one which is 'free'

(henceforth referred to as 'the free ROM bank on GoMMC'). To the machine, the

free ROM bank is no different than any other ROM bank : it is read-only, its

contents are remembered when the machine is powered down, it appears as ROM

number x (more about 'x' below). Its contents can only be changed by special

means (the GoMMC software). I.e. it is similar to a flash ROM.

Refer to 'Sideways ROM/RAM' for more detailed information (bank numbers etc.).

Note also that GoMMC can be 'read disabled', to prevent corrupted code in one

or both of GoMMC's ROM banks from crashing the machine. See 'Recovery' for more

information.

//

//

// The software

//

//

The GoMMC software consists of several components :

- A sideways ROM providing basic facilities

- A set of tools for the '8-bit' environment

- A set of tools for the (IBM compatible) PC environment

The sideways ROM provides low level access to the MMC, interfaces with patched

filing systems and provides simple interactive functionality (select and use

discs). The tools mainly allow discs and other data to be added to the MMC.

They are mostly command-line tools, to allow operation in batch mode (e.g.

from an *EXEC file).

The 8-bit tools provide for :

- Formatting the MMC (MMCFO)

- Performing a sideways ROM update (MMCUP)

- Adding floppies (discs) to the MMC (MMCADxx)

- Adding patched filing systems to the MMC (MMCAFS)

- Adding tools to the MMC (MMCATO)

- Wiping the GoMMC free ROM bank (MMCWI)

- Writing discs back from MMC to floppy (MMCFlop)

- Quick interactive selection of discs (MMCMenu)

- A 'rescue' ROM image (TOOLROM)

The PC tools provide for :

- Add/extract/delete/rename discs/tools to/from the MMC (GoMMCio.exe)

GoMMCio.exe runs under Windows XP (or compatible). An executable suitable for

Linux is provided in the form of GoMMCio.unx (best renamed to e.g. 'gommcio'

under Linux).

The functionality provided by the sideways ROM is discussed in the sections

titled 'GoMMC ROM : etc.'. The tools are discussed in the sections titled

'Tools : etc.'.

//

//

// Filing systems

//

//

Probably the most 'difficult' part of using GoMMC is how to handle (patched)

filing systems.

Filing systems reside in sideways ROM or RAM. It is highly recommended that you

first read 'Sideways ROM/RAM', to get to know more about the sideways ROM/RAM

banks in your machine.

Regular/original filing systems like DFS and ADFS cannot access the MMC 'out of

the box'. Most of them were meant to (and can only) access physical floppies,

via a disc interface (based on an 8271 or 1770 floppy disc controller).

Regular/original filing systems can, however, be patched to access the MMC (via

GoMMC) instead of physical floppies (note that such a patched filing system can

no longer access physical floppies !). In GoMMC parlance, these filing systems

are simply called 'patched filing systems'. Wherever in this guide I refer to a

'patched filing system', such a filing system is meant, and not the original.

A patched filing system can be introduced to the system in EPROM (insert this

into any free sideways ROM socket), or by auto-loading it into either the free

ROM bank on GoMMC, or a sideways RAM bank (see *MMCConfig FSNR and FSRM for

more info).

The GoMMC software (MMCAFS in particular, in this respect) provides suitable

patches for a number of filing systems. Refer to 'Supported filing systems' for

more details.

Although, at some stage, you usually need to run both the original and patched

versions of a filing system, the first to be able to copy the contents of your

physical floppies to the MMC, the second to be able to access them from there,

original and patched versions of the same filing system cannot be run together

at the same time.

On the BBC Master, the mechanism to arrange this is simple. When you need to

add things to the MMC, ensure you've *INSERTed the original version of the

filing system, and *UNPLUGged the patched version of the filing system (if

any). When you wish to access those discs via GoMMC, ensure you've *INSERTed

the patched version, and *UNPLUGged the original version.

On the BBC B/BBC B+/Electron, the GoMMC ROM software can help (this is only

100% reliable when GoMMC runs from ROM socket 15). To (in BBC Master terms)

'*Unplug' a ROM, use

*MMCConfig ROMx 1

where 'x' is the ROM number (0 through 9 and A through F for ROMs 0 through 15

respectively). To (again in BBC Master terms) '*Insert' a ROM, use

*MMCConfig ROMx 0

Then press CTRL-BREAK. The 'unplugged' ROM will now be inactive and the

'inserted' one will be active.

NOTE !!! If the original and patched filing system do not differ in their first

1K bytes, the OS will permanently disable the lowest numbered ROM of the two

(currently, this is the case for, e.g., DFS 2.26). In such a case, call the

higher numbered one 'x' and the lower numbered one 'y', and then not only

*MMCConfig ROMx 1

but *also*

*MMCConfig ROMy 2

(i.e. instead of 0). This ensures the lower numbered ROM is not disabled by the

OS (or, rather, re-enabled by GoMMC).

Note that GoMMC is perfectly happy to serve multiple filing systems at the same

time. E.g. it's perfectly possible to run both DFS 2.26 and ADFS 1.30 at the

same time. Just ensure that the discs you select with *MMCDisc have the proper

(DFS or ADFS) type. Accessing a DFS disc with a patched ADFS or accessing an

ADFS disc with a patched DFS is not recommended, although rarely, if at all,

fatal (ADFS will complain about a 'Broken directory', DFS will list garbage

with e.g. *CAT).

//

//

// Tools : 8-bit environment

//

//

The 8-bit tools MMCUP, MMCFO, MMCADDG, MMCADAG, MMCADDI, MMCADAI, MMCAFS,

MMCATO and MMCWI run on your BBC or Electron only. They are all command-line

utilities, some taking arguments, some not. They all need to have their load

and execution addresses set to (hex) &2000 (check that with *INFO before you

run them).

In addition, there is a tool called 'MMCFlop' with which you can write any disc

on the MMC back to a floppy, and a tool called 'MMCMenu', an interactive disc

selection program. Both are BASIC programs. Simply CHAIN them.

You can store these tools on a physical floppy and run them via an unpatched

filing system, or you can store them on the MMC and run them via the *MMCTool

command, which eliminates the need to have a physical floppy, with the tools on

it, accessible at all times.

Finally, the TOOLROM file contains a ROM image. Refer to 'Tools : TOOLROM'.

//

//

// Tools : PC environment

//

//

A powerful tool called GoMMCio.exe is provided, allowing you to manipulate the

MMC contents on a PC running Windows XP or (x86) Linux, with, attached to the

machine (e.g. via USB), a flash memory card reader that accepts MMCs.

//

//

// Tools : MMCUP

//

//

This tool updates the GoMMC ROM software.

Usually, a previous version of the software will already be running on your

machine. If so, its sideways ROM number will be determined automatically. If

not, you will be asked for the number of the sideways ROM socket that GoMMC is

in (0 through 9 and A through F specify slots 0 through 15 respectively).

After running MMCUP, always switch off the machine, then switch it on again.

//

//

// Tools : MMCFO

//

//

This is the first tool you will/must use on the MMC. It formats the MMC for use

by GoMMC.

Note that all the usual formatting warnings apply, especially that all data

previously stored on the MMC is lost, so use this tool with care.

After running MMCFO, always press CTRL-BREAK.

//

//

// Tools : MMCAFS

//

//

This tool allows you to add (GoMMC-) patched filing systems to the MMC, so they

can be auto-loaded (refer to the discussion of *MMCConfig FSNR and FSRM).

Type '*MMCAFS <file name>' where <file name> is the file name of the original,

i.e. unpatched (!) filing system ROM image (it must be exactly 16K bytes in

size). If the filing system image is recognized, it is patched, and added to

the MMC. Refer to 'Supported filing systems' for a list.

Specifying a filename of 'MMCFSx' (where x is the GoMMC internal filing system

number) accepts the image face value (instead of checking its CRC against the

known value). This allows slightly tweaked FS images to be used (at your own

risk). Just be very sure of the correct 'x' to use.

Specifying a filename of 'MMCFSy' (where y is >= 81) is similar to specifying

'MMCFSx', but the image is not patched as well, i.e. the original filing system

is stored. This has a different internal filing system number than the patched

one (80 + x where x is the patched one), so it can exist alongside it. When you

make use of the filing system auto-load function (*MMCConfig FSNR) its presence

on the MMC can be exploited to more easily switch between original and patched

filing systems (simply reconfigure to the other one and *MMCReset (or, if that

fails, power-cycle the machine) to auto-load it).

//

//

// Tools : MMCATO

//

//

This tool allows you to add (GoMMC) tools to the MMC, so they can be run

directly from the MMC, without a disc containing the tools (or the filing

system needed to access it) present.

Type '*MMCATO <file name> <tool name>' to add tool <file name> under the name

<tool name>. Henceforth, you can execute that tool via *MMCTool (refer to its

discussion elsewhere).

Note that a tool may be a BASIC program as well as a machine code program, and

need not be a standard GoMMC tool : it may also be something you made yourself.

For more details, refer to the discussion of *MMCTool.

//

//

// Tools : MMCADxx

//

//

These tools (MMCADDG, MMCADAG, MMCADDI, MMCADAI) allow you to add discs to the

MMC. A.k.a. 'what it's all about' ! ;-)

These tools take either a disc image (i.e. an image of a disc in a file) or a

physical floppy as their input. Disc image files may be on any filing system

you want. You only need to ensure that the file is accessible when you run the

tool.

Type e.g. '*MMCADDG <file name> <disc name>' (all other tools have the same

syntax) to add a disc image in file <file name> under name <disc name> (the

latter is what you henceforth specify to *MMCDisc and *MMCList to find the

disc).

The disc name may be up to 48 characters long. Note, in this respect, that you

can use a simple kind of 'directory' structure (refer to the discussion of

*MMCPrefix), so it pays to put a little thought in how to name discs.

MMCADDG (Add Disc DFS Generic) is used to add DFS type discs, with either a

disc image file, or a physical floppy, as the source. A disc image may be any

size, up to 800 K (for a dual-drive, double-sided disc image), must contain

<drive 0> <drive 2> <drive 1> <drive 3>, in that order, and results in a disc

of the same size as the disc image (but rounded up to a 512 byte multiple). To

copy a physical floppy to the MMC, specify $1$, $2$ or $3$ as the <file name>,

to copy, respectively, a single single-sided (200K), a single double-sided

(400K), or double double-sided (800K) floppy (i.e. two floppies in two drives

!).

MMCADDG also accepts $4$. This is like $1$ but copies from drive 2 instead of

drive 0. MMCADDG also accepts $5$ through $8$, which are exactly like $1$

through $4$, resulting in the same total disc size on the MMC, but copy only

the first 40 tracks of every physical drive (the last 100K of every drive on

the MMC therefore becomes unused and unusable space). Use $5$ through $8$ to

copy physical 40 track discs only.

MMCADAG (Add Disc ADFS Generic) is used to add ADFS type discs, with either a

disc image file, or a physical floppy, as the source. A disc image may be any

size, up to 1280 K (for a dual-drive, double-sided disc image), must contain

<drive 0> <drive 1>, in that order, and results in a disc of the same size as

the disc image (but rounded up to a 512 byte multiple). To copy a physical

floppy to the MMC, specify $1$, $2$ or $3$ as the <file name>, to copy,

respectively, a single single-sided (320K), a single double-sided (640K), or

double double-sided (1280K) floppy (i.e. two floppies in two drives !).

MMCADAG also accepts $4$. This copies the entire ADFS disc in drive 0 (be it

floppy or harddisc). I.e. it enables straight-to-MMC copying of harddiscs. A

floppy may just as well be copied with $1$ or $2$.

MMCADDI (Add Disc DFS Interleaved) is used to add DFS type discs, with an

'interleaved' disc image file as the source. An 'interleaved' disc image is

always double-sided, must contain <track 0, drive 0> <track 0, drive 2> <track

1, drive 0> ... <track 79, drive 2>, in that order, and results in a 400K DFS

type disc.

MMCADAI (Add Disc ADFS Interleaved) is used to add ADFS type discs, with an

'interleaved' disc image file as the source. An 'interleaved' disc image is

always double-sided, must contain <track 0, head 0> <track 0, head 1> <track 1,

head 0> ... <track 79, head 1>, in that order, and results in a 640K ADFS type

disc.

Note that adding from physical floppy switches to DFS/ADFS and drive 0, so if

you do not run your tools via *MMCTool, remember to switch back to the FS/drive

your tools are on after adding every disc !

//

//

// Tools : MMCWI

//

//

MMCWI wipes the free ROM bank on GoMMC. Refer to 'Recovery' for more details.

//

//

// Tools : MMCFlop

//

//

This tool allows you to write discs on the MMC back to a (physical) floppy.

Ensure you're running version 0.54 or higher of the GoMMC ROM software. Also

ensure that you have disabled all patched filing systems (use '*Unplug' on the

BBC Master, or '*MMCConfig ROMx 1' on the BBC B/BBC B+/Electron).

Simply CHAIN "MMCFlop". It will present a simple menu :

0. Exit

1. Copy DFS disc

2. Copy ADFS disc

3. Switch to drive 1 (now 0)

Press 0,1,2 or 3.

0 exits.

1 will copy a disc on the MMC to a DFS type floppy. You must run an unpatched

(!) DFS type filing system that can be selected by *DISC. Simply enter the disc

name, and the disc will be copied back to floppy (if the disc is a 4-drive one,

to two floppies, one in drive 0/2, one in drive 1/3).

2 will copy a disc on the MMC to an ADFS type floppy. You must run an unpatched

(!) ADFS type filing system that can be selected by *ADFS. Simply enter the

disc name, and the disc will be copied back to floppy (if the disc is a 2-drive

one, to two floppies, one in drive 0, one in drive 1).

3 will toggle between drive 0 and 1 as the (first) output drive. Do not copy

multiple-drive discs with '1' selected as the current drive.

//

//

// Tools : MMCMenu

//

//

This tool allows you to interactively select discs. It is meant to be used as a

'disc menu' type of program, in conjunction with the BOOT configuration option

(refer to its discussion elsewhere).

When it is run, you're prompted to enter (any part of) a disc name to search

for (you can make use of the left and right arrow keys and DELETE, and SPACE

will reset/wipe the disc name). Press RETURN to start the search. All matches

that '*MMCList *name*' would have brought up (where 'name' is what has been

entered of the disc name) will be listed. Choose any one of them by pressing

the key indicated in front of the full disc name, or press RETURN to return to

the disc name prompt and do another search. If there are more matches than will

fit on screen, the list will end with a '...' line. Enter more of the disc name

to reduce the number of matches, or choose a disc anyway, as described.

Choosing a listed disc name will execute an *MMCDisc <disc name> command and

exit MMCMenu. Assuming a patched filing system is running, subsequently

pressing SHIFT-BREAK should then boot the selected disc.

MMCMenu is ideally used in conjunction with a non-zero BOOT setting. E.g. if

you put MMCMenu on the MMC as tool '!Boot' (using MMCATO), and set BOOT to 1

(using MMCConfig), every CTRL-BREAK will auto-load MMCMenu (you then select a

disc, as described) and the subsequent SHIFT-BREAK will boot the selected disc

(MMCMenu won't be run this time because BOOT is implicitly zero when SHIFT is

pressed). Or put MMCMenu on a DFS disc, create a !Boot file containing 'CHAIN

"MMCMenu"', use *OPT 4 3 on the disc, and set BOOT to 2 (then go : CTRL-BREAK,

!Boot disc now selected, SHIFT-BREAK, MMCMenu now loads, and SHIFT-BREAK again

after interactively selecting a disc).

//

//

// Tools : TOOLROM

//

//

This is a ROM image that contains a copy of MMCUP, MMCWI, MMCFO and MMCATO,

allowing you to run them, from ROM, via the ROM filing system (standard on all

BBCs and Electrons).

It may be useful if you need (but are unable) to run MMCUP or MMCFO on your

machine in any other way (e.g. when your only means of file transport from the

outside world to your BBC or Electron is GoMMC, and the GoMMC ROM software has

become corrupted).

Burn the ROM image into an (E)PROM. Then : power down, insert the ROM into a

free sideways ROM socket inside your machine, power up, enter '*ROM', then

enter e.g. '*RUN MMCUP'.

Note that on an Electron, you can run the ToolROM by (temporarily) switching

the utility socket on the interface to 'ROM' mode (see the GoMMC installation

instruction pages on Internet), and replacing whatever is in the utility socket

with the ToolROM.

!!! VERY IMPORTANT !!! On a BBC model B or Electron, always before *RUNning

something from the tool ROM, enter '!&C8=&70' (or, if you wish, '!200=112').

This is necessary in order to circumvent a serious bug in OS 1.20 when *RUNning

files under the ROM filing system.

E.g. to run MMCUP you would do :

*ROM

!&C8=&70

*RUN MMCUP

//

//

// Tools : GoMMCio.exe

//

//

GoMMCio.exe allows batch-wise addition, extraction, deletion and renaming of

discs and tools, including backup/restore of the entire MMC contents, on a PC

with a flash memory card reader, running Windows XP, or an 'x86' Linux.

GoMMCio.exe accepts a script file containing commands (the 'add' commands are

identical to the ones you'd use to add discs and tools on your Acorn machine),

and 'executes' these commands all at once.

The syntax is : 'GoMMCio.exe -d<drive> <script>', where <drive> is the drive

letter under which the MMC appears via the flash card reader (if you don't know

which one it is, the best way to find out is to click on all the drives via

Explorer, and find the one that complains about an 'unformatted' drive), and

<script> is the filename of the script. Example : 'GoMMCio.exe -dK script.txt'.

Under Linux, use e.g. '/dev/sdc' (whatever your MMC appears as under /dev)

instead of the drive letter.

You can optionally add a '-f' to the command-line. Use -f with extreme caution

because it instructs GoMMCio.exe to 'forget' about the old contents of the MMC

before doing anything. Note that the MMC must have been formatted (with MMCFO),

although it need not be empty. If the MMC hasn't been formatted before, you can

use '-x' together with '-f' to force a temporary '1 MB MMC' format (use this

only in the course of transferring MMCFO to your machine if there is no other

way to do that than via the MMC and GoMMCio.exe, and don't forget to run MMCFO

immediately afterwards to properly format the MMC !).

Add '-l' to the command-line to list the new MMC catalogue after the script has

been executed.

Add '-s' to the command-line to make GoMMCio shrink (if possible) single-drive

non-interlaced DFS or ADFS discs (added with MMCADDG or MMCADAG) while writing

them to the MMC. This attempts to remove all free space from the drive, and its

total size will be reduced accordingly (both on the MMC and as registered in

the drive's catalogue). You will not be able to add files or directories to the

disc (at least, not without deleting others), but it will occupy the absolute

minimum of space on the MMC.

Add '-b<size>' (where <size> is e.g. '200K' or '10M') to the command-line to

make GoMMCio 'blow up' single-drive non-interlaced DFS or ADFS discs (added

with MMCADDG or MMCADAG) to the specified size while writing them to the MMC.

The maximum size is 200 KB or 512 MB for DFS or ADFS discs respectively.

To execute a single command instead of a script, use '-c' followed by a script

command between double quotes (e.g. '-c "NEWADFS 10M hd1"').

A script is a text file (e.g. 'script.txt' as for the example) that looks like

(everything between the '---' only) :

---

# start

MMCADDG C:\GoMMC\Discs\Elite.ssd Elite

MMCADDG C:\GoMMC\Discs\Hopper.ssd Hopper

MMCADDG C:\GoMMC\Discs\Snapper.ssd Snapper

# end

---

There's one command per line, empty lines are ignored, and if it starts with a

'#' it is a comment and is ignored as well. This particular script would simply

add three discs.

You can use the following commands :

- MMCADDG, MMCADAG, MMCADDI, MMCADAI and MMCATO : all of them have the syntax

and semantics of their 8-bit counterparts (refer to 'Tools : MMCADxx' and

'Tools : MMCATO' for more information), except that the $x$ type arguments

are not available.

- MMCAFS <file name> MMCFSx (where x is the GoMMC internal filing system

number) restores a backupped patched filing system (see BACKUP).

- MMCEXDG, MMCEXAG, MMCEXDI, MMCEXAI : are like their 'add' counterparts, but

copy the disc from the MMC to a file instead of the other way around.

- BACKUP <directory name> copies the entire contents of the MMC to the given

directory, in the form of seperate files (each containing one object) and a

script. All formerly 'deleted' objects are discarded. To restore a backup,

simply run the script through GoMMCio.exe.

- DELTOOL <tool name>, DELDISC <disc name>, and DELFS <fs number> will delete

the specified tool, disc or patched filing system respectively. It is only

marked as 'deleted' on the MMC. To actually reclaim the space that deleted

objects occupied (e.g. if your MMC is getting full), do a BACKUP, and, from

the backup directory, run the backup script through GoMMCio.exe (don't forget

a '-f' on the command line).

- RENTOOL <old tool name> <new tool name> and RENDISC <old disc name> <new disc

name> will rename the specified tool or disc respectively.

- NEWDFS <disc size> <disc name> creates an empty DFS disc of anything between

16 KB and 800 KB in size (e.g. '16K' or '800K' for <disc size>). A size above

200/400/600 KB results in 2/3/4 drive discs respectively.

- NEWADFS <disc size(s)> <disc name> creates an empty ADFS disc of anything

between 16 KB and 512 MB in size (e.g. '16K' or '512M' for <disc size(s)>),

or 16+16 KB and 512+512 MB ('16K+16K' or '512M+512M' for <disc size(s)>).

That is, the non-+ form creates single drive discs, the +-form double drive

discs (note that the two drive sizes may be completely different).

- MMCATAP and MMCEXTA add and extract, respectively, tape images. They are like

their 'disc' counterparts, e.g. MMCADDG and MMCEXDG, except that MMCATAP

expects an 'ungzipped' UEF file (as available from Internet). GoMMCio.exe

puts a 'stripped' version of this UEF file on the MMC. MMCATAP will recognize

and accept this stripped version as well (e.g. when restoring a BACKUP, since

this will have saved the stripped versions, not the original UEFs). Refer to

'Tapes' for more details.

//

//

// Tapes

//

//

You can load 'tapes' via GoMMC as well, via the standard *TAPE filing system.

Currently this works on BBC model Bs running OS 1.20 only.

The BBC's OS ROM must be patched to achieve this. Which means you must burn a

patched OS ROM image into a suitable EPROM, and replace your OS ROM with that

EPROM. The patches in the OS are needed to make the standard *TAPE filing

system get input from GoMMC, instead of the tape hardware.

As already indicated, currently there is only a patch for BBC model B's OS

1.20. The patched image (to be blown into a 27(C)128 EPROM) can be obtained

here :

http://web.inter.nl.net/users/J.Kortink/home/hardware/gommc/os/POS120.BIN

You can add tape images to the MMC using GoMMCio.exe's MMCATAP command, using

'ungzipped' UEF files (as available from Internet) as source material. They can

subsequently be selected by *MMCDisc, and 'played' under the tape filing system

(if it is not selected, type '*TAPE'). Simply issue the usual CHAIN"" or *RUN

command to start loading the 'tape'. Use *MMCRewind to rewind the 'tape' (note

that *MMCDisc implicitly rewinds as well).

Note that, when you download UEF files from Internet, you may have to 'unzip'

the downloaded file (to obtain the actual .uef file) as well as 'ungzip' the

.uef file (e.g. rename it to a .uef.gz, then 'gzip -d' it, after obtaining a

copy of gzip).

Note that you can 'slow down' tape loading with the SLOW configure option. See

its description for more details.

//

//

// GoMMC ROM : Star commands

//

//

Nine *-commands are available from the GoMMC sideways ROM software (type *Help

GoMMC to list them) :

- *MMCInfo

Displays general information. Currently this is the GoMMC version number and

date, and storage statistics : the total, used and free number of MMC bytes,

and the number of objects on the MMC.

- *MMCDisc (<name>)

Selects disc <name> for subsequent access (by the currently selected patched

filing system). The usual * and # wildcards may be used (the first matching

disc is selected). If <name> is not specified, '*' is implied.

ALWAYS press CTRL-BREAK after selecting a disc with *MMCDisc !

- *MMCExtra (<name>)

Is like *MMCDisc, but allows selection of a second, separate disc (the one

selected with *MMCDisc being the 'first' and main one).

Under ADFS the second disc is accessed for drives 4 through 7, while the first

one is accessed for drives 0 through 3 (but note that, in practice, drives

2,3,6 and 7 don't exist and are mapped to drives 0,1,4 and 5 respectively).

Under DFS the second disc is like a seamless extension of the first. E.g., if

both are 400K discs (i.e. 2-drive), the first one is accessed for drives 0 and

2, and the second for drives 1 and 3, as if there was one 800K (i.e. 4-drive)

disc. So, metaphorically speaking, this particular arrangement (2 400K discs)

allows independent selection of 'the disc in the first floppy drive' (using

*MMCDisc) and 'the disc in the second floppy drive' (using *MMCExtra). Other,

more esoteric combinations of disc sizes are fine, keeping in mind that it's

like having the two discs concatenated into one (e.g. for two 200K discs, the

first will become drive 0, the second drive 2).

- *MMCList (<name>)

Lists all discs (and/or other objects, see *MMCConfig LTYP) matching <name>.

The usual * and # wildcards may be used. If <name> is not specified, '*' is

implied.

- *MMCPrefix (<name>)

Affects *MMCDisc, *MMCExtra, *MMCList and *MMCTool. The 'name prefix' may be

set by this command. The name prefix is what implicitly precedes the <name>

specified in *MMCDisc, *MMCExtra, *MMCList and *MMCTool. If <name prefix> is

not specified, the name prefix is cleared (this is also the default setting).

E.g. suppose you have three discs, named 'Acornsoft.Hopper', 'Acornsoft.Elite'

and 'Aardvark.Frak'. Setting *MMCPrefix Acornsoft. (note the trailing dot !)

would cause *MMCDisc Hopper to select the first disc, and *MMCList * to list

the Acornsoft discs only (as if you had typed *MMCDisc Acornsoft.Hopper, and

*MMCList Acornsoft.*).

NOTE : a leading '$' in the name specified to *MMCDisc, *MMCExtra, *MMCList and

*MMCTool will skip matching the prefix (and the '$' is discarded). E.g. if the

prefix is 'A.', then the name '$!Boot' matches '!Boot' only, not 'A.!Boot'.

- *MMCConfig (<name> <value>)

Allows setting (and listing) configuration options. Refer to 'GoMMC ROM :

Configuration' for more information.

- *MMCReset

Performs a hard reset (useful after, e.g., an *MMCConfig FSNR/FSRM, avoids

having to physically switch off/on the power supply).

- *MMCRewind

Rewinds the currently selected 'tape'. Refer to 'Tapes' for more details.

- *MMCTool <name> ...

Runs tool <name>. The usual * and # wildcards may be used (the first matching

tool is run).

Tools added to the MMC (using the MMCATO tool) can be run with *MMCTool. Where

you'd have typed e.g. '*MMCADDG filename discname' you can now type '*MMCTool

toolname filename discname', where toolname is the tool name you specified to

MMCATO.

Tools run with *MMCTool are always copied to &2000. Tools starting with an &0D

byte are considered BASIC, and are executed by inserting 'PAGE=&2000<CR>OLD

<CR>RUN<CR>' into the keyboard buffer. All other tools are considered machine

code, and are executed by calling &2000 *after* poking a 2-byte pointer to the

command line tail to &2003 (low byte) and &2004 (high byte) (so there better be

(e.g.) a JMP &2005 at &2000).

Note that you can auto-execute an *MMCTool command on every BREAK using the

BOOT configuration option (refer to its discussion elsewhere).

//

//

// GoMMC ROM : Configuration

//

//

'*MMCConfig (<name> <value>)' configures the GoMMC ROM software. *MMCConfig on

its own, with no arguments, displays a list of the currently configured values.

The <value> is always a decimal number from 0 to 255. The <name> is one of :

FSNR : sets the auto-loaded filing system number (see below)

FSRM : sets the auto-loaded filing system sideways ROM number (see below)

F2NR : sets the (2nd) auto-loaded filing system number (see below)

F2RM : sets the (2nd) auto-loaded filing system sideways ROM number (see below)

LOPT : when set to 1 causes *MMCList to show 'extended' info (see below)

LTYP : indicates which types of object *MMCList shows (see below)

BOOT : sets the boot option (see below)

SLOW : sets the artificial delay used in tape loading

WPRO : sets the MMC write protection level

TUBE : selects '1-byte' or '2-byte' (or 'warp') Tube transfers

ROM0 : when set to 1 will cause GoMMC to attempt to 'unplug' ROM number 0

ROM1 : when set to 1 will cause GoMMC to attempt to 'unplug' ROM number 1

....

ROME : when set to 1 will cause GoMMC to attempt to 'unplug' ROM number 14

ROMF : when set to 1 will cause GoMMC to attempt to 'unplug' ROM number 15

The ROMx options also have a setting 2, to force the OS to 'see' the ROM in

question. Refer to 'Filing systems' for more details.

After a software upgrade (with MMCUP), all values default to 0.

- FSNR and FSRM (F2NR and F2RM for the second filing system, if any)

FSNR and FSRM combined configure the filing system auto-load function. With

this, a filing system can be auto-loaded, on powerup, into either the free ROM

bank on GoMMC, or a sideways RAM bank (as long as it is writable simply by

normal selection, most simple and some complex systems for the BBC B/B+, the

BBC Master SWR system, and the Electron (if RAM is installed on the GoMMC for

Electron interface) qualify for this).

Refer to 'Sideways ROM/RAM' for details specific to your machine.

FSNR can be :

0             - auto-load disabled

1 and higher  - auto-load filing system number 1 and higher respectively

81 and higher - auto-load filing system number 81 and higher respectively

255           - auto-wipe (load 0's)

FSRM can be :

0-15 - select ROM bank 0-15 respectively

See 'Supported filing systems' for filing system numbers.

After configuring FSNR to a supported filing system number, *MMCReset or

power-cycle the machine and the filing system will be auto-loaded. Note that

the filing system must be present on the MMC (put there by the MMCAFS tool,

refer to its discussion elsewhere in this guide).

After configuring FSNR to 255, *MMCReset or power-cycle the machine and the

selected bank will be wiped (to all 0's). Note that this is really only useful

for non-volatile banks, like the free ROM bank on GoMMC, and then only if you

want or need to make the system forget whatever is loaded in there.

- LOPT and LTYP

LOPT and LTYP 'tweak' *MMCList. In 'normal' mode (LOPT=0), *MMCList shows just

the object name. In 'extended' mode (LOPT=1), it shows 'tt ss aaaaaaaa +

ssssssss name', all values in hex, where tt is the object type, ss the object

subtype, aaaaaaaa the start address on the MMC, ssssssss the size in bytes. In

addition, LTYP selects the object types to be shown, set bit x in the value to

show type x in the listing, types are 1 through 3 for patched filing systems,

tools, and discs respectively (i.e. LTYP=14 (or e.g. 255) would list them all).

LTYP=0 is like LTYP=8 (i.e. list discs only).

- BOOT

BOOT, when set to 1, will, on every powerup or BREAK, insert '*MMCTool $!Boot'

into the keyboard buffer. I.e. this is kind of an '*OPT 4 2' (as available from

most filing systems) for your MMC. Press SHIFT during powerup or BREAK to

suppress the boot action (e.g. in case your !Boot malfunctions). BOOT = 0 is

'off'. BOOT = 2 will execute an '*MMCDisc $!Boot' (see discussion of MMCMenu

for a useful application). BOOT = 33 through 126 will execute an '*MMCTool

$!Bootx', where x is the ASCII character corresponding to the BOOT value (e.g.

65 would result in '$!BootA'). All other non-zero values act as value 1. Note

that the '$' is skipped and causes the *MMCPrefix setting to be ignored. Refer

to the discussion of *MMCPrefix for more information.

- SLOW

The value of SLOW is the artificial delay used in returning 'tape bytes'. 0

through 255 matches 'no delay' through maximum delay. You may want to use a

non-zero value if the tape loading speed is to appear more 'realistic'.

- WPRO

WPRO, when set to anything else but 0, will write-protect the MMC. If WPRO = 1,

write protection is 'silent'. If WPRO = 2, an error will be generated when an

attempt is made to write to the MMC. All other non-zero values act as value 2.

- TUBE

TUBE, when set to 1, uses normal '1-byte' transfers across the Tube (i.e. when

you have your second processor switched on, and you are transferring files to

or from its memory). Set to 2, '2-byte' transfers are used, which are (almost)

twice as fast. 0 acts as 1, higher than 2 acts as 2. In most cases you can use

2, but if you experience problems (which is highly unlikely though), simply

switch 'back' to 1.

Set TUBE to 255 to select the experimental (possibly unreliable !) 'warp' mode.

This seems to work ok on ARM7TDMI coprocessors, and reaches dizzying heights of

roughly 150 KB/sec. But do not trust this setting while working with valuable

data.

- ROM0 through ROMF

The ROMx 'configures' allow you to disable/re-enable filing systems, especially

useful if you have both a patched and an unpatched version of the same filing

system in your machine (in that case, you must ensure one and only one of them

is 'plugged in' at any one time !). Refer to 'Filing systems' for more details.

Note that for the ROM unplug function to work, GoMMC should ideally be in ROM

socket 15 on the BBC B or Electron, or 10/11 on the BBC B+. On the BBC Master,

there is no need to use the ROM unplug functions of GoMMC, because *Unplug and

*Insert can then be used instead. Remember to always press CTRL-BREAK after

unplugging or inserting ROMs.

//

//

// Sideways ROM/RAM

//

//

Different machines have different sideways ROM/RAM systems :

The BBC model B

---------------

The standard machine has four sideways ROM sockets and no sideways RAM.

Starting at the ROM socket closest to the side of the machine, going 'west',

the socket numbers are : 15, 14, 13, 12.

The socket that the GoMMC 'adapter' is plugged into (see the GoMMC installation

instruction pages on Internet), is the socket where the free ROM bank on GoMMC

appears. This can be considered to be a sideways ROM bank, whose number may

nevertheless be specified to *MMCConfig FSRM.

The BBC Master

--------------

The standard machine has three sideways ROM sockets (two 32 KB, one 16 KB), and

four sideways RAM banks. Although not all at the same time, see below.

Starting at the ROM socket that is closest to the cartridge connector(s), going

'north', the socket numbers are : 4+5, 8, 6+7. 4+5 and 6+7 are both 32 KB

sockets (i.e. two ROM/RAM banks each). Whether they are sideways ROM banks

(i.e. reflect what is in the actual socket) or sideways RAM banks (what is in

the actual socket is ignored) is determined by links. If LK18 is set 'west',

4+5 is sideways RAM (else, i.e. 'east', ROM). If LK19 is set 'west', 6+7 is

sideways RAM (else, i.e. 'east', ROM). LK19 is always 'east', for GoMMC.

GoMMC is in socket number 6, and socket number 7 is the socket where the free

ROM bank on GoMMC appears. This can be considered to be a sideways ROM bank,

whose number may nevertheless be specified to *MMCConfig FSRM.

The BBC B+

----------

The standard machine has five sideways ROM sockets (all either 16 or 32 KB,

selectable via links), and no sideways RAM.

ICs 35, 44, 57, 62, 68 respectively, are socket numbers 2+3, 4+5, 6+7, 8+9 and

10+11 respectively, and links 9, 11, 12, 15 and 18 respectively determine

whether they are 32 KB sockets (link set 'east') or 16 KB sockets (link set

'west').

If GoMMC is in socket x+y, GoMMC proper is in x, and y is the socket where the

free ROM bank on GoMMC appears. This can be considered to be a sideways ROM

bank, whose number may nevertheless be specified to *MMCConfig FSRM.

The Electron

------------

You are using the 'GoMMC for Electron' interface. This offers one of :

a) No banks (no chip is installed in the utility socket)

b) Up to 2 sideways ROM banks (28-pin ROM installed in the utility socket)

c) Up to 2 sideways RAM banks (28-pin SRAM installed in the utility socket)

d) Up to 8 sideways ROM banks (32-pin ROM installed in the utility socket)

e) Up to 8 sideways RAM banks (32-pin SRAM installed in the utility socket)

f) 2 sideways FRAM banks (Ramtron FM1808 installed in the utility socket)

FRAM banks are like RAM banks, but are non-volatile as well (their content is

remembered after power down). GoMMC itself uses FRAM (although it is writable

only by special means, unlike the banks for option f), which can be written to

freely).

See the GoMMC installation instruction pages on Internet for further details,

including how to install the chip.

The bank numbers are a little odd. In cases b), c) and f), the lower bank is 3

(and mirror images appear in 2, 1 and 0), and the higher bank is 13 (and mirror

images appear in 12, 7 and 6). In cases d) and e), going from lower to higher

addresses within the chip, the banks are 1, 13, 3, 7, 0, 12, 2, 6 (this is, of

course, only relevant if you use a ROM, and need to know where to put the ROM

images within the chip's address space).

GoMMC is in socket number 15, and socket number 14 is the socket where the free

ROM bank on GoMMC appears. This can be considered to be a sideways ROM bank,

whose number may nevertheless be specified to *MMCConfig FSRM.

//

//

// Supported filing systems

//

//

GoMMC supports (and provides patches for) the following filing systems :

------------------------------------

Code  Name

------------------------------------

1  (BBC B) Acorn DNFS 1.20

2  (BBC B) Acorn DFS 2.26

3  (BBC B) Acorn ADFS 1.30

4  (BBC Master) Acorn DFS 2.24

5  (BBC Master) Acorn ADFS 1.50

*  6  (BBC Master) Acorn DFS 2.45

*  7  (BBC Master) Acorn ADFS 2.03

8  (BBC B) Watford DFS 1.44

!  9  (BBC B) ACP DFS 1.00

! 10  (Electron) ACP DFS 2.20

11  (Electron) Acorn ADFS 1.00

------------------------------------

The code is the GoMMC 'internal' filing system number, i.e. as specified to

*MMCConfig FSNR and listed as the subtype in 'extended' *MMCLists.

Note that numbers 81 and higher may be used as well. These are the original,

unpatched versions of the patched filing systems whose number is 80 less (refer

to the discussion of MMCAFS).

* : note that DFS 2.45 and ADFS 2.03 are co-dependent (they share routines) and

need to be in the machine at the same time, one in an even numbered ROM slot x,

the other in ROM slot x + 1. If you auto-load one of them (with FSNR/FSRM), use

F2NR/F2RM to auto-load the other (i.e. in the Master, you'll have to use slots

4 and 5).

! : note that these are 'E00' filing systems (they leave the default PAGE in an

unexpanded machine at &0E00), and have to run in a sideways RAM bank (note that

the free ROM bank on GoMMC is not suitable).

//

//

// GoMMC ROM : CRC check

//

//

The GoMMC ROM software checks itself for corruption on every powerup (including

after *MMCReset) using a CRC check.

If this check fails, 'GoMMC : ROM corrupted !' will be displayed at the top of

the screen, and you're advised to follow the instructions under 'Recovery'.

Thanks to Greg Cook for providing an especially fast implementation of the CRC

algorithm.

//

//

// GoMMC ROM : Error messages

//

//

The following is an exhaustive list of error messages issued by the GoMMC ROM

software :

'Please switch off Tube' (code 147) : *MMCTool will not run code on your second

processor, so please switch it off.

'Select a GoMMC disc' (code 183) : No disc has been selected yet with *MMCDisc

or *MMCExtra.

'Outside GoMMC disc' (code 183) : An attempt was made to access data 'outside'

the current disc space.

'MMC is write-protected' (code 201) : An attempt was made to write to the MMC

while it was write-protected.

'Bad GoMMC FS' (code 0) : The patch used on the patched filing system you are

attempting to run has become obsolete. Update the patch.

'Syntax error: <correct syntax>' (code 220) : A syntax error has been found in

a *-command.

'Escape' (code 17) : Escape was pressed during *MMCList.

In addition, these non-BRK warnings/errors are issued on a (hard) reset :

'GoMMC : MMC doesn't respond' : the MMC doesn't seem to be present.

'GoMMC : ROM corrupted !' : your GoMMC ROM software has been corrupted. Refer

to 'GoMMC ROM : CRC check' for further information.

'GoMMC : Failed to auto-load FS' : while trying to auto-load a filing system,

either it cannot be found on the MMC, or you are trying to load it in into the

ROM slot occupied by the GoMMC ROM.

//

//

// Recovery

//

//

If the software stored on GoMMC (either the GoMMC ROM, or the free ROM bank, or

both) has become corrupted, your machine may hang on power up, and no recovery

may seem possible.

Fear not. This problem has been foreseen. GoMMC can be switched to 'write only'

mode, so the corrupted software can no longer hang the machine, but can still

be overwritten. MMCUP (to restore the GoMMC ROM) and MMCWI (to wipe the free

ROM bank) are the tools to use.

Follow the following recipe :

1) Power down.

2) Move the three-legged jumper on GoMMC (just below the MMC socket) to the

'west' or 'left' position (away from the PCB edge).

3) Power up. Your machine should no longer hang, but GoMMC has 'gone' as well.

4) Run MMCUP. See 'Tools : MMCUP' for details.

5) Run MMCWI and answer the questions (e.g. answer 6 and 7 respectively for the

BBC Master).

6) Power down.

7) Move the three-legged jumper on GoMMC back to the 'east' or 'right' position

(nearest the PCB edge).

8) Power up. Your machine should no longer hang, and GoMMC is back in action.

//

//

// What is remembered on power-down

//

//

GoMMC makes use of writable but nevertheless non-volatile memory called FRAM.

This (aside from the other possibility, to store it on the MMC) allows GoMMC to

'remember' some user settings, even after power-down.

These are :

- All settings done via *MMCConfig

- The currently selected disc

- The *MMCPrefix setting

=======

===========

===============

Signing off

===============

===========

=======

//

//

// Epilogue

//

//

For any remaining questions, please send email to : kortink@inter.nl.net.

John Kortink, 17 December 2006